PROBLEM SOLUTION LIGHT/Darkness THE 2 SOLUTION!!!!

This solution is similarly however for it is not constant the MOBILEPHONE in the power mode.

An addition transistor must be inserted only with light on power mode switches.

It is a PNP transistor BC 560, the connection

E (emitter at resistance),

B (base at Led)

C (Collector at mass).

If the light turns on sets it the resistance against mass.

One could refine the thing now with a SMD transistor.

One knows each PNP - transistor for it using!

Good Luck!!!

PROBLEMLÖSUNG HELL/DUNKEL

Handy aufschrauben...

2. DC/DC-IC suchen (befindet sich auf der Rückseite der Platine oberhalb des SIM-Karten-Anschlusses und unterhalb der Kondensatorenreihe). Rechts unterhalb des DC/DC IC's befindet sich ein weiterer kleiner IC (6 Beine), links und rechts von diesem zwei Widerstände.

3. Kurze Funktionsbeschreibung:

Der DC/DC-Converter wandelt die Akkuspannung des Handys in eine durch eine Widerstandsbeschaltung außerhalb des IC's festgelegte  Spannung. Bei der Umschaltung in den "HIGH POWER" Mode werden einfach noch zwei Widerstände zugeschalten, somit ändert sich die externe Widerstandsbeschaltung und der DC/DC- Wandler liefert eine andere Spannung (3,4V bzw 3,6V). Es sind nun einfach die beiden Widerstände dauerhaft zuzuschalten.

4. Vorgehensweise:

Wie im Bild unten zu sehen muss eine Brücke zwischen den beiden Widerständen und Masse eingelötet werden. Wie immer nur mit großer Lupe und feinem Lötkolben zu bewältigen.

TRANSLATE IN ENGLISH WITH BABLEFISH /ALTAVISTA

PROBLEM SOLUTION FLASHING BLUE LEDS

OPEN THE MOBILEPHONE...

2nd DC/DC IC on look up (is on the rear side of the circuit board above the SIM card link and below the condenser series). On the right below the DC/DC IC's is a further small IC (6 legs), on the left and on the right from this two of resistances.

3. Short function description: The DC/DC converter changes the Akkuspannung of the Handys into a voltage determined by a resistance wiring outside of the IC's. During switching into the " HIGH POWER " mode become simply still two resistances course-switch, thus change the external resistance wiring and DC/CDc transducer supply another voltage (3,4V or 3,6V). There is now easily both resistances permanently to connect   the 4th methodology: As in the picture down to see a bridge between the two resistances and mass must be soldered. To be mastered as in each case with large magnifying glass and fine soldering iron.

Hier gibt es Blau Led´s günstig !!!!

DIE FOLGENDEN BILDER MIT E 61 CHIP VERBINDUNG SIND NUR DA FÜR DA

WENN MAN SEINE TASTATUR DURCH HERUMLÖTEN AN DEN & PINS GEKILLT HAT !!!

ES BEWIRKT ! NUR DAS DIE TASTATUR WIEDER FUNKTIONIERT !!

Einfach ein Isoliertes dünnes Kabel von dem E61 D zu dem aussenring der Menütaste löten !!!! GOOD LUCK !!!!!

BETANKT EUCH MAL BEI PTNB DAFÜR !!!!!!!!!!!!

Also hier eine kleine Kurzanleitung um das 3210 gut auseinander zu bekommen ! ! !

Here are the instructions to change the green back lights to blue back lights ! ! !

1. FIRST LOOSEN THIS SCREW

2. THEN REMOVE THIS SCREW

3. THEN WITH A SMALL SCREWDRIVER CAREFULLY LEVER UPWARD BETWEEN ANTENNA COVER AND HOUSING

4. THEN DO THE SAME ON THE OTHER SIDE HOWEVER STILL BEING CAREFUL

THEN SHIFT THE COVER UPWARDS

6. CAUTION: DO NOT BREAK THE SMALL TAB ON TOP THAT HOLDS IT ALL TOGETHER

7. THEN YOU CAN LOOSEN THIS SCREW UNDER THE COVER

8. AND THEN THE LAST SCREW

9. AND NOW TAKE THE MOBILE PHONE APART CAREFULLY

10. GOOD LUCK

Und es geht und wie hier der beste beweis ! ! !

Ihr müst nur ein ruhiges Händchen und viel geduld haben ( EIn Lötkolben mit Nadel Spitze und eine Monster Lupe)

Und dieses Bild unten damit ihr nicht die Led`s falschrum auflötet .

Und immer daran denken nicht erst 1 Blaue Led auflöten und testen dann tut sich gar nichts erst die 6 von der Tastatur

dann könnt ihres schon antesten und dann die 4 vom Display !

And the most important part follows! You must have a calm hand and much patience (also a soldering iron with a needle point and a large magnifying glass) and follow the picture below to ensure all LEDs are soldered in the right direction. Soldering 1 blue LED and testing will not work at all. Only when all 6 of the keyboard LEDs are replaced can they be tested, then the 4 from the display!

THANKS Dominik Bloemhard for the English Translate ! ! ! ! !

Danke Frank für die Klasse Led bschreibung

Hier gibt es die Blauen LED`s